| ID | DESC |
| --- | --- |
| SCU_STC_SRS_585 | The STC Program shall enter and remain in Failsafe Level 2 when a failure is reported by any one of the following Monitors: LVDT Double Loss Monitor, ASA Jump Monitor Status, LVDT No Angle Found Fault, EHSV1 and EHSV2 Monitor, Target Input Monitor, Over Angle Monitor, Watchdog Test Monitor, Watchdog Maximum Reset Count, ROM Test, RAM Integrity test, Rigging record and data Check, Aircraft Information check Note1: Each of these individual Monitors is defined as non-recoverable, therefore ensuring the STC Program cannot leave Failsafe level 2 after any one of these has been reported. Note2: The ROM Test and Ram Test are classified as a recoverable Monitors in the Supplier Equipment Specification (Ref : B2) however this is only after a reset of the Control Lane. Note3: Some of the above fault conditions result in processor resets which eventually end up with the processor stopping entry into a run-time mode. The SCU then behaves as if it is in Failsafe Level 2 because, firstly, the affected processor disables the SOV and secondly, the other processor will be in Failsafe Level 2 because of Loss of Interlane Communications. Object Type: Traceable Reqt Source: SES Verification Method: |
| SCU_STC_SRS_811 | The STC Program shall determine A_DMC_9 LADC Monitor Status according to the following logic: IF the following condition is true for 220ms with a tolerance of +10ms (( No valid A_DMC_9 LADC labels have been received ) OR (A_DMC_9 LADC data have wrong (even) parity bit) OR (A_DMC_9 LADC have SSM data set to "Normal Operation")) THEN A_DMC_9 LADC Monitor Status = TRUE ELSE IF the following condition is true for 320ms with a tolerance of +10ms ((A_DMC_9 LADC labels have been received at expected update rate with a tolerance of +10ms ) AND (A_DMC_9 LADC data have correct (odd) parity bit) AND (A_DMC_9 LADC have SSM data set to "Normal Operation") AND (A_DMC_9 LADC Airspeed < 35kts)) THEN A_DMC_9 LADC Monitor Status = FALSE ELSE No action (A_DMC_9 LADC Monitor Status remains unchanged) ENDIF ENDIF Where A_DMC_9 LADC ARINC Message is defined in SCU_STC_SRS_135 Justification for timing tolerance: ARINC label reception requires tolerance for label polling and processing. Object Type: Traceable Reqt Source: SES Verification Method: |
| SCU_STC_SRS_99 | The STC Program shall convert the Pilot_SHDL_Operation into a steering angle demand Unlimited_Pilot_SHDL_Command in every Major cycle using the following logic: IF( Pilot_SHDL_Operation >= 54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (26.4 * Pilot_SHDL_Operation - 840.5deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation - 298.58deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 34.7deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation - 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation - 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= 13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation - 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= 3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation - 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -3deg ) THEN Unlimited_Pilot_SHDL_Command = 0deg ELSE IF( Pilot_SHDL_Operation >= -13.3deg ) THEN Unlimited_Pilot_SHDL_Command = (Pilot_SHDL_Operation + 3deg ) * (6.8 / 10.3) ELSE IF( Pilot_SHDL_Operation >= -23.7deg ) THEN Unlimited_Pilot_SHDL_Command = (9.2 * Pilot_SHDL_Operation + 51.64deg ) * (1/10.4) ELSE IF( Pilot_SHDL_Operation >= -34deg ) THEN Unlimited_Pilot_SHDL_Command = (11.6 * Pilot_SHDL_Operation + 110.12deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -44.3deg ) THEN Unlimited_Pilot_SHDL_Command = (14.4 * Pilot_SHDL_Operation + 205.32deg ) * (1/10.3) ELSE IF( Pilot_SHDL_Operation >= -54.7deg ) THEN Unlimited_Pilot_SHDL_Command = (16.6 * Pilot_SHDL_Operation + 298.58deg ) * (1/10.4) END IF Where Pilot_SHDL_Operation is the value calculated in SCU_STC_SRS_68 and Major cycle is defined in Table 6.2-1 Object Type: Traceable Reqt Source: SES Verification Method: |
| SCU_STC_SRS_180 | The STC Program shall debounce the NLG_WOW1 signal using the following algorithm to generate a filtered version of the signal IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = FALSE ELSE IF (NLG_WOW1 has been FALSE for the last 100ms) THEN Debounced_NLG_WOW1 = TRUE ELSE Debounced_NLG_WOW1 remains unchanged, with an initial value of FALSE ENDIF Where NLG_WOW1 is the value calculated in SCU_STC_SRS_139 Object Type: Traceable Reqt Source: SES Verification Method: |
| SCU_STC_SRS_137 | The STC Program shall poll the ARINC receivers for the labels listed in the following table more frequently than the transmission frequency: Table: ARINC Rx Rate Object Type: Traceable Reqt Source: SATA-15-166 Verification Method: ARINC Receiver; Label (octal); Transmission Interval (ms). DMCA1; 022; 40.96. DMCA1; 226; 1400. DMCA1; 174 (Right); 100. DMCA1; 206 (ISI); 67. DMCA1; 206 (L ADC); 200. DMCA1; 206 (R ADC); 100. DMCA1; 260; 1000. DMCA1; 107; Aperiodic (polling rate 1000). DMCB1; 022; 40.96. DMCB1; 226; 1400. DMCB1; 174 (Right); 100. DMCB1; 206 (ISI); 67. DMCB1; 206 (L ADC); 100. DMCB1; 206 (R ADC); 200. DMCB1; 260; 1000. ACE1B1; 136; 100. ACE1B2; 136; 10. |
| SCU_STC_SRS_135 | The STC Program shall receive A_DMC_9 L ADC Airspeed in the Computed Airspeed field in messages from DMCA in the format of Label 206 (ADC) specified in below Table with a Source/Destination Identifier set to Left ADC Object Type: Traceable Reqt Source: SATA-15-166 Verification Method: Bit Position; Field; Description; Definition. 8-1; Label; Octal Label – ARINC429 Message ID; Label = 206 (octal) Bit 1=msb, Bit 8=lsb. 10-9; SDI; Source/Destination Identifier; [00] = Not Used [01] = Left ADC [10] = Right ADC [11] = Not Used. 28-12; Computed Airspeed; Aircraft Speed as calculated by the Air Data Computer (ADC); Range: 0 to 1023.99609375 Units: Knots LSB Resolution: 0.00390625 kts MSB Value: 512 kts Bit 28 = msb; Bit 11 = lsb. 29; Sign; Sign; [0] = Positive [1]= Negative. 31-30; SSM; Sign / Status Matrix Type: BNR (Numeric); [00] = Failure Warning [01] = No Computed Data [10] = Functional Test [11] = Normal Operation. 32; Parity; Parity Bit (Odd); Set to ‘Zero’ or ‘One’ in order to make the total number of Ones in the message an Odd value.. |
| SCU_STC_SRS_100 | The STC Program shall prevent the use of A_DMC_9 LADC ARINC data for downstream processing by detecting and isolating invalid data when any of the following conditions persist for 220 ms with a tolerance of +10 ms: • No valid A_DMC_9 LADC labels are received • A_DMC_9 LADC data contain incorrect (even) parity • A_DMC_9 LADC SSM is set to “Normal Operation” incorrectly The STC Program shall isolate invalid or unsafe A_DMC_9 LADC ARINC data by forcing the A_DMC_9 LADC Monitor Status to remain asserted while any invalid data condition exists. The STC Program shall inhibit restoration of normal operation until valid A_DMC_9 LADC data integrity conditions are continuously satisfied for at least 320 ms with a tolerance of +10 ms, including correct label reception rate, correct parity, valid SSM, and airspeed less than 35 kts |
| SCU_STC_SRS_101 | The STC Program shall continuously monitor the A_DMC_9 LADC ARINC data for integrity issues. It checks for missing labels, incorrect parity, and unexpected SSM values over a 220 ms period. Alerts are logged when invalid data are detected, but no action is specified to prevent or isolate it. |
| SCU_STC_SRS_102 | Invalid A_DMC_9 LADC ARINC data must be addressed immediately. No further details are given about how the STC Program should respond when these conditions occur. Operators are informed through the system interface |
| SCU_STC_SRS_103 | The STC Program shall isolate invalid A_DMC_9 LADC ARINC data as appropriate. It normally prevents downstream processing when labels are missing or parity is incorrect. Data integrity is logged for review, and operators may be notified. |
| SCU_STC_SRS_104 | The STC Program may isolate invalid A_DMC_9 LADC ARINC data when detected. It should alert operators if downstream systems might be affected by faulty data. The STC Program will log all incidents for post-flight analysis. |
| SCU_STC_SRS_200 | The STC Program shall isolate invalid A_DMC_9 LADC data if parity errors are detected. This action ensures that downstream systems continue to operate safely |
| SCU_STC_SRS_201 | The STC Program shall debounce the NLG_WOW1 signal using a filtered algorithm and shall generate a TRUE/FALSE output based on signal persistence. IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = FALSE ELSE Debounced_NLG_WOW1 = TRUE ENDIF |
| SCU_STC_SRS_202 | The STC Program shall debounce the NLG_WOW1 signal and monitor the NLG_WOW2 signal for integrity. IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = FALSE ELSE Debounced_NLG_WOW1 = TRUE ENDIF |
| SCU_STC_SRS_203 | The STC Program shall debounce and calculate the filtered NLG_WOW1 signal output. IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = FALSE ELSE Debounced_NLG_WOW1 = TRUE ENDIF |
| SCU_STC_SRS_204 | The STC Program shall debounce the NLG_WOW1 and NLG_WOW2 signals using a filtered algorithm. IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = FALSE ELSE Debounced_NLG_WOW1 = TRUE ENDIF |
| SCU_STC_SRS_205 | The STC Program shall debounce the NLG_WOW1 signal using a filtered algorithm. IF (NLG_WOW1 has been TRUE for the last 1.5s) THEN Debounced_NLG_WOW1 = FALSE ELSE IF (NLG_WOW1 has been FALSE for the last 100ms) THEN Debounced_NLG_WOW1 = TRUE ELSE Debounced_NLG_WOW1 remains unchanged ENDIF |
| SCU_STC_SRS_300 | The STC Program shall initialize the NLG_WOW1 signal filter at system startup. It immediately begins monitoring the NLG_WOW1 signal to ensure proper debounce operation |
| SCU_STC_SRS_301 | The STC Program will perform the Pilot and Co-Pilot S-HDL Steering command calculation and limiting and the Rudder Command calculation. This will be followed up by the Steering Demand summation and limiting function. A Touchdown Protection function will be implemented in order to prevent steering at high speed |
| SCU_STC_SRS_302 | The STC Program shall perform the Pilot and Co-Pilot S-HDL Steering command calculation and shall limit the Rudder Command output. This will be followed up by the Steering Demand summation and limiting function. A Touchdown Protection function will be implemented to prevent steering at high speed. |
| SCU_STC_SRS_303 | Pilot and Co-Pilot S-HDL Steering command calculation and limiting are performed by the STC Program. The Rudder Command calculation is then completed, followed by the Steering Demand summation and limiting function. A Touchdown Protection function is implemented in order to prevent steering at high speed. |
| SCU_STC_SRS_304 | The STC Program will perform the Pilot and Co-Pilot S-HDL Steering command calculation and limiting and the Rudder Command calculation. This will be followed up by the Steering Demand summation and limiting function. A Touchdown Protection function will be implemented to prevent steering at high speed |
| SCU_STC_SRS_305 | The STC Program shall perform the Pilot and Co-Pilot S-HDL Steering command calculation and limiting and the Rudder Command calculation as appropriate. This is followed by the Steering Demand summation and limiting function. A Touchdown Protection function shall be implemented with adequate WOW sensor debouncing |
| SCU_STC_SRS_306 | The STC Program shall perform the Pilot and Co-Pilot S-HDL Steering command calculation and limiting and the Rudder Command calculation. This shall be followed up by the Steering Demand summation and limiting function. A Touchdown Protection function shall be provided to prevent steering at high speed. |
| SCU_STC_SRS_400 | The STC Program shall sample analogue input values from the LVDT and RVDT interfaces. The resulting DFT calculations shall be used to determine the SHDL positions and the actual LVDT/RVDT positions SACT. Discrete input signals shall be sampled and debounced for further processing, and ARINC 429 input labels for WOW status, Airspeed, and Rudder Pedal Position shall be validated. |
| SCU_STC_SRS_401 | The STC Program will sample analogue input values from the LVDT and RVDT interfaces. The resulting DFT calculations will be used to determine the SHDL positions. |
| SCU_STC_SRS_402 | The STC Program shall debounce discrete input signals for further processing as appropriate. ARINC 429 input labels for WOW status and Airspeed shall be validated |
| SCU_STC_SRS_403 | The STC Program shall perform a robust check on ARINC 429 input labels for WOW status, Airspeed, and Rudder Pedal Position. Invalid labels shall be flagged for further processing |
| SCU_STC_SRS_500 | The STC Program shall monitor flash memory, DSP registers, and system components for faults, and check received and transmitted ARINC data for validity. |
| SCU_STC_SRS_99 | The STC Program shall monitor flash memory, DSP registers, and system components for faults. Received and transmitted ARINC data shall be checked for validity through loopback comparison. |
| SCU_STC_SRS_504 |  |
| SCU_STC_SRS_607 | The STC Program shall determine EHSV2 Monitor Status with a confirmation time of 200ms according to the following logic: IF (EHSV2 Monitor Status = FALSE) THEN EHSV2 Monitor Status = Debounced_EHSV2_Fail ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Debounced_EHSV2_Fail is defined in SCU_STC_SRS_164 from Raw_EHSV2_Fail defined in SCU_STC_SRS_124 |
| SCU_STC_SRS_608 | The STC Program shall determine EHSV1 and EHSV2 Monitor Status according to the following logic: IF (EHSV1 and EHSV2 Monitor Status = FALSE) THEN EHSV1 and EHSV2 Monitor Status = (EHSV1 Monitor Status) AND (EHSV2 Monitor Status) ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where EHSV1 Monitor Status is defined in SCU_STC_SRS_606 and EHSV2 Monitor Status is defined in SCU_STC_SRS_607 |
| SCU_STC_SRS_609 | The STC Program shall determine EHSV2 Monitor Status with a confirmation time of 200ms according to the following logic: IF (EHSV2 Monitor Status = FALSE) THEN EHSV1 Monitor Status = Debounced_EHSV2_Fail ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Debounced_EHSV1_Fail is defined in SCU_STC_SRS_164 from Raw_EHSV2_Fail defined in MRJ_SCU_STC_SRS_124 |
| SCU_STC_SRS_610 | The STC Program shall determine EHSV1 Monitor Status with a confirmation time of 200ms according to the following logic: IF (EHSV1 Monitor Status = FALSE) THEN EHSV2 Monitor Status = Debounced_EHSV1_Fail ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Debounced_EHSV1_Fail is defined in SCU_STC_SRS_164 from Raw_EHSV1_Fail defined in SCU_STC_SRS_124 |
| SCU_STC_SRS_606 | The STC Program shall determine EHSV1 Monitor Status with a confirmation time of 200ms according to the following logic: IF (EHSV1 Monitor Status = FALSE) THEN EHSV1 Monitor Status = Debounced_EHSV1_Fail ELSE do nothing (monitor status remains TRUE, making it Non-recoverable) ENDIF Where Debounced_EHSV1_Fail is defined in SCU_STC_SRS_164 from Raw_EHSV1_Fail defined in SCU_STC_SRS_124 |